附件1：安全数据单样本（中文）                                   报验号：37000010192102104第5页，共5页

化学品安全数据单

一、标识

	全球统一制度产品标识符：丙炔醇/ Propargyl alcohol。
其它标识办法：/
化学品使用建议和使用限制：/ 

供货商的详细情况：河南海源精细化工有限公司。

供货商地址：河南省濮阳市台前县产业集聚区
紧急电话号码：0393-2603883


二、危险标识
	物质或混合物的分类：

易燃液体类别3。急毒性(口服)类别3。急毒性(皮肤)类别3。急毒性(吸入)类别3。皮肤腐蚀/刺激类别1B。严重眼损伤/眼刺激类别1。危害水生环境(慢性)类别2。
全球统一制度标签要素，包括防范说明：

[image: image1.png]




 INCLUDEPICTURE "http://115.28.146.172:8050/GHSweb1/DQhtml1/images/XXX5.png" \* MERGEFORMATINET [image: image2.png]




 INCLUDEPICTURE "http://115.28.146.172:8050/GHSweb1/DQhtml1/images/XXX4.png" \* MERGEFORMATINET [image: image3.png]


 [image: image4.png]



信号词：危险。

危险说明：易燃液体和蒸气。吞咽会中毒。皮肤接触会中毒。吸入会中毒。造成严重皮肤灼伤和眼损伤。对水生生物有毒并具有长期持续影响。
防范说明：

预防：
远离热源、热表面、火花、明火和其他点火源。禁止吸烟。保持容器密闭。容器和装载设备接地并等势联接。使用防爆的[电气/通风/照明/……]设备。使用不产生火花的工具。采取防止静电放电的措施。戴防护手套/穿防护服/戴防护眼罩/戴防护面具/戴听力防护装置。作业后彻底清洗……。使用本产品时不要进食、饮水或吸烟。不要吸入粉尘/烟/气体/气雾/蒸汽/喷雾。只能在室外或通风良好处使用。避免释放到环境中。

反应：
万一起火，使用干粉、抗溶性泡沫、雾状水、二氧化碳灭火。如误吞咽：立即呼叫中毒急救中心/医生。具体治疗（见下文）。漱口。不得诱导呕吐。如皮肤（或头发）沾染：立即脱掉所有沾染的衣服，用水清洗皮肤/淋浴。如感觉不适，呼叫中毒急救中心/医生。具体治疗（见下文）。沾染的衣服清洗后方可重新使用。如误吸入：将人转移到空气新鲜处，保持呼吸舒适体位。立即呼叫中毒急救中心/医生。具体治疗（见下文）。如进入眼睛：用水小心冲洗几分钟。如戴隐形眼镜并可方便地取出，取出隐形眼镜。继续冲洗。立即呼叫中毒急救中心/医生。收集溢出物。

贮存：
存放于通风良好处。保持低温。存放处须加锁。保持容器密闭。
处置：
按照相关规定处置内装物和容器。
不导致分类的其他危险：/


三、组成/成分信息
	化学名称
	化学文摘社登记号码（CAS No.）
	含量%

	丙炔醇
	107-19-7
	99.96


四、急救措施
	必要的急救措施
吸入：如果吸入，请将患者移到新鲜空气处。如果停止了呼吸，给予人工呼吸。求医。
皮肤接触：用肥皂和大量的水冲洗。求医。
眼睛接触：用大量水彻底冲洗至少15分钟。就医。

食入：如果吞食，禁止催吐。切勿给失去知觉者从嘴里喂食任何东西。用水漱口。就医。
最重要的急性和延迟症状/效应：/
必要时注明立即就医及所需的特殊治疗：/


五、消防措施
	适当的灭火介质：泡沫，干粉，二氧化碳。喷水或水雾-仅适于大火。
化学品产生的具体危险：液体和蒸气易燃。受热或接触明火，有中等程度的火灾危险。蒸气与空气能形成爆炸性混合物。受热或接触明火，有中等程度的爆炸性危险。蒸气可能会飘散到离火源相当远的地方。受热能引起膨胀或分解，导致容器急剧破裂。燃烧时可能释放有毒的一氧化碳(CO)烟雾。
消防人员的特殊防护行为：仅在火灾时，佩戴呼吸设备及防护手套。采取一切可能的措施防止溢出物进入下水道或水道。采用适合于周围环境的灭火程序。不要靠近可能灼热的容器。从有防护的位置喷水以便冷却暴露于火灾中的容器。如果这么做安全的话，将容器从火场中移走。


六、意外释放措施
	人身防范、保护设备和应急程序：清除所有点火源。立即清理所有泄漏物。避免吸入蒸气，避免接触皮肤和眼睛。使用防护装备控制人员接触。
环境防范措施：在安全的前提下，阻止泄漏。
抑制和清理的方法和材料：小量泄漏：用蛭石或其它吸附性物质来收集并吸附少量泄漏物。擦除。大量泄漏：可以使用喷水或水雾来驱散/吸收蒸气。用沙子、土或蛭石来吸收泄漏物。只能使用不产生火花的铲子和防爆设备。收集可回收的产品于贴有标签的容器中，以便回收利用。用沙子、土或蛭石来吸收残留物。收集固体残留物，密封于贴有标签的桶中，以便废弃处理。冲洗沾染区域，防止废水排入阴沟。


七、搬运与储存
	安全搬运的防范措施：防止所有接触，包括吸入。当有接触危险时，穿戴防护服。在通风良好的区域使用。防止本品在低洼处汇集。未作空气检测，禁止进入封闭空间内。禁止接触人体、食品或食品容器。避免接触不相容物料。操作处置时，禁止进食、饮水或吸烟。不使用时，保持容器安全密封。防止容器受到物理损伤。
安全存储的条件，包括任何不相容性：在批准的易燃液体储存区存放。禁止吸烟，避免强光直射以及远离火源。保持容器严实密封。存放于阴凉、干燥且通风良好的地点，远离不相容物质。保护容器以避免受到损害，并定期检查是否有漏洞。遵守生产商的储存和处置建议。


八、接触控制/人身保护
	控制参数：

适当的工程控制：对易燃液体和易燃气体，可能需要局部通风系统或工艺围栏通风系统。通风设备应防爆。
个人保护措施
防护眼罩/面具：带侧边的安全护目镜。化学护目镜。
皮肤防护：戴化学防护手套(如聚氯乙烯 PVC)。穿安全鞋或安全靴(如橡胶材料)。
呼吸系统防护：充足容量的A种过滤器。

高温危险：/


九、物理及化学性质
	外观（物理状态、颜色等）
气味
气味阙值
pH值
熔点/凝固点
初始沸点和沸腾范围
闪点
蒸发速率
易燃性（固态、气态）
上下易燃极限或爆炸极限
蒸气压力(kPa)

蒸气密度(空气 = 1)
相对密度(水 = 1)

可溶性
分配系数：n-辛醇/水
自动点火温度
分解温度

粘度(cSt)
	无色透明液体
/
/
/
-48～-52℃

114°C

35.0°C
/
易燃

空气中3.4%～70%（体积）

20℃时1.54kPa

1.93
0.97
混溶
/
/
/
/


十、稳定及反应性
	反应性：/
化学稳定性：当物质的存储条件具有足够的稳定剂/聚合抑制剂时，物质性质稳定。高温下储存或长期储存能引起聚合和固化。大量物质，如200升的桶装物能积聚热量，释放出高度刺激的沸腾蒸气。

严禁打开正在升温的热储存桶--用水从外部冷却以防止蒸气释放。
危险反应的可能性：与强氧化剂激烈反应。浸蚀许多塑料。在加热、氧化剂和过氧化物作用下，该物质可能聚合。
应避免的条件：高温、热源、点火源等。

不相容材料：与强酸、酰基氯、酸酐、氧化剂和还原剂不相容。
危险分解产物：二氧化碳(CO2)、一氧化碳(CO)、 有机物燃烧产生的其他典型热解产物。


十一、毒理学信息
	暴露途径：吸入、经口、皮肤、眼睛。

有关物理、化学和毒理学特点的症状：/

急性毒性效应：
吸入本物质在正常生产过程中生成的蒸气或气溶胶(雾、烟)，可对身体产生毒害作用。意外食入本物质可能引起毒害作用。食入本物质可对口腔和胃肠道造成化学灼伤。皮肤接触本品可能有害，吸收后可导致全身性反应。皮肤直接接触本物质可造成化学灼伤。眼睛直接接触该物质可造成化学灼伤。

慢性毒性或长期毒性效应：反复或长期接触腐蚀性物质，可能导致牙齿腐蚀、口腔炎症和溃疡以及颌骨坏死(少见)。可能引起伴有咳嗽的支气管刺激症状以及支气管肺炎频繁发作。还可能会发生胃肠功能紊乱。长期接触可能引起皮炎和/或结膜炎。
毒性的数值度量（如急性毒性估计值）： 
吸入（鼠）LC50: 2000mg/m3/2H


十二、生态信息

3

	16mg/L



	持久性及降解性：水/土壤：低。空气：低。
生物累积潜力：低 (LogKOW = 0.38)。
在土壤中的流动性：低 (KOC = 1.325)。
其它有害效应：/


十三、处置考虑
	处置方法：尽可能回收本物质。如果不能确定有合适的处理或废弃处置设备，联系制造商有关回收方法，或联系当地或地区的废物管理部门有关废弃方法。按如下方法废弃处理：在有许可证的填埋处进行掩埋或在有许可证的焚化场进行焚化(与适当的可燃物质混合后)。对空的容器进行去污处理。遵守所有的标注规定，直至容器被清洗或销毁为止。


十四、运输信息
	联合国编号：2929。
联合国运输名称：有机毒性液体，易燃，未另作规定的。
运输危险种类：6.1+3。
包装类别：II。
环境危害：/
使用者的特殊防范措施：/


十五、管理信息
	国内化学品安全管理法规：

本化学品安全技术说明书遵照了以下相关国家标准：GB/T16483-2008，GB13690-2009， GB18218-2009，GB15258-2009，GB6944-2012，GB190-2009，GB/T191-2008，GB12268-2012，GA57-1993，GB/T 15098-2008，GBZ 2-2007以及相关法规：《危险货物运输管理规则》、《危险化学品安全管理条例》。


十六、其它信息
	参考文献
	联合国《关于危险货物运输的建议书.规章范本》

联合国《全球化学品统一分类和标签制度》


	制表日期
	2019年04月03日


注1：当产品为含有两种以上危险物质的混合物时，应依据其混合后的危险性，制作安全数据单。

注2：制造商/供应商应根据实际情况确保安全数据单所含信息的正确性，并适时更新。

注3：如由于产品特性而不存在或不可得某些信息时（如固体不存在沸点），应在表格中以“/”标识。  

                        Chemical Safety Data Sheet
SECTION 1  IDENTIFICATION
	GHS Product identifier: Propargyl alcohol.
Other means of identification: /
Recommended use of the chemical and restrictions on use: /
Supplier’s details: /HENAN HAIYUAN CHEMICAL CO.,LTD

Detailed address：INDUSTRIAL AGGLOMERATION AREA OF TAIQIAN COUNTY,PUYANG CITY,HENAN PROVINCE
Emergency phone number:0393-2603883


SECTION 2  HAZARDS IDENTIFICATION

	Classification of the substance or mixture

Flammable Liquid Category 3, Acute Toxicity (Oral) Category 3, Acute Toxicity (Dermal) Category 3, 
Acute Toxicity (Inhalation) Category 3, Skin Corrosion/irritation Category 1B, Serious Eye Damage Category 1, Hazardous to the aquatic environment, long term hazard Category 2.
GHS Label elements, including precautionary statements
[image: image5.png]




 INCLUDEPICTURE "http://115.28.146.172:8050/GHSweb1/DQhtml1/images/XXX5.png" \* MERGEFORMATINET [image: image6.png]




 INCLUDEPICTURE "http://115.28.146.172:8050/GHSweb1/DQhtml1/images/XXX4.png" \* MERGEFORMATINET [image: image7.png]


[image: image8.png]



Signal word: Danger
Hazard statement(s): Flammable liquid and vapor. Toxic if swallowed. Toxic in contact with skin. Toxic if inhaled. Causes severe skin burns and eye damage. Toxic to aquatic life with long lasting effects.

Precautionary statement(s):
Prevention: 

Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. Keep container tightly closed. Ground and bond container and receiving equipment. Use explosion-proof [electrical/ventilating/lighting/...] equipment. Use non-sparking tools. Take precautionary measures against static discharge. Wear protective gloves/protective clothing/eye protection/face protection. Wash … thoroughly after handling. Do not eat, drink or smoke when using this product. Use only outdoors or in a well-ventilated area. Do not breathe dust/fume/gas/mist/ vapors / spray. Avoid release to the environment.

Response:

In case of fire: Use foam, dry chemical powder, carbon dioxide, water spray to extinguish. IF SWALLOWED: Immediately call a POISON CENTER/doctor. Specific treatment (see below). Rinse mouth. Do NOT induce vomiting. IF INHALED: Remove person to fresh air and keep comfortable for breathing. Specific treatment (see below). Immediately call a POISON CENTER/doctor. IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower. Call a POISON CENTER/doctor if you feel unwell. Specific treatment (see below). Wash contaminated clothing before reuse. IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a POISON CENTER/doctor. Collect spillage..

Storage: 

Store in well-ventilated place. Keep cool. Store locked up. Keep container tightly closed.

Disposal: 

Dispose of contents/container in accordance with local/regional/national/international regulations.

Other hazards which do not result in classification: /


SECTION 3  COMPOSITION/INFORMATION ON INGREDIENTS

	Chemical Name
	CAS No.
	Concentration%

	Propargyl alcohol
	107-19-7
	99.96


SECTION 4  FIRST AID MEASURES

	Description of necessary first aid measures
If inhaled: If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.
In case of skin contact: Wash off with soap and plenty of water. Consult a physician.
In case of eye contact: Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.
If swallowed: If swallowed do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.
Most important symptoms and effects, both acute and delayed: /

Indication of immediate medical attention and special treatment needed：/


SECTION 5  FIREFIGHTING MEASURES

	Suitable extinguishing media: Foam. Dry chemical powder. Carbon dioxide. Water spray or fog - Large fires only.
Special hazards arising from the chemical: Liquid and vapour are flammable. Moderate fire hazard when exposed to heat or flame. Vapour forms an explosive mixture with air. Moderate explosion hazard when exposed to heat or flame. Vapour may travel a considerable distance to source of ignition. Heating may cause expansion or decomposition leading to violent rupture of containers. On combustion, may emit toxic fumes of carbon monoxide (CO).

Special protective actions for fire-fighters: Wear breathing apparatus plus protective gloves in the event of a fire. Prevent, by any means available, spillage from entering drains or water courses. Use fire fighting procedures suitable for surrounding area. DO NOT approach containers suspected to be hot. Cool fire exposed containers with water spray from a protected location. If safe to do so, remove containers from path of fire.


SECTION 6  ACCIDENTAL RELEASE MEASURES

	Personal precautions, protective equipment and emergency procedures: Remove all ignition sources. Clean up all spills immediately. Avoid breathing vapours and contact with skin and eyes. Control personal contact with the substance, by using protective equipment.
Environmental precautions: Stop leak if safe to do so.
Methods and materials for containment and cleaning up: Minor Spills: Contain and absorb small quantities with vermiculite or other absorbent material. Wipe up. Major Spills: Water spray or fog may be used to disperse / absorb vapour. Contain spill with sand, earth or vermiculite. Use only spark-free shovels and explosion proof equipment. Collect recoverable product into labelled containers for recycling. Absorb remaining product with sand, earth or vermiculite. Collect solid residues and seal in labelled drums for disposal. Wash area and prevent runoff into drains.


SECTION 7  HANDLING AND STORAGE

	Precautions for safe handling: Avoid all personal contact, including inhalation. Wear protective clothing when risk of exposure occurs. Use in a well-ventilated area. Prevent concentration in hollows and sumps. DO NOT enter confined spaces until atmosphere has been checked. DO NOT allow material to contact humans, exposed food or food utensils. Avoid contact with incompatible materials. When handling, DO NOT eat, drink or smoke. Keep containers securely sealed when not in use. Avoid physical damage to containers.
Conditions for safe storage, including any incompatibilities: Store in approved flammable liquid storage area. No smoking, naked lights/ignition sources. Keep containers securely sealed. Store away from incompatible materials in a cool, dry, well-ventilated area. Protect containers against physical damage and check regularly for leaks. Observe manufacturer's storage and handling recommendations contained within this SDS.


SECTION 8  EXPOSURE CONTROLS/PERSONAL PROTECTION

	Control parameters: /

Appropriate engineering controls: For flammable liquids and flammable gases, local exhaust ventilation or a process enclosure ventilation system may be required. Ventilation equipment should be explosion-resistant.
Personal protective equipment
Eye/face protection: Safety glasses with side shields. Chemical goggles. 
Skin protection: Wear chemical protective gloves, eg. PVC. Wear safety footwear or safety gumboots, eg. Rubber.
Respiratory protection: Type A Filter of sufficient capacity.

Thermal hazards: /


SECTION 9  PHYSICAL AND CHEMICAL PROPERTIES

	Appearance
Odour

Odour Threshold
pH

Melting point/freezing point
Initial boiling point and boiling range
Flash point

Evaporation rate

Flammability (solid, gas)
Upper/lower flammability or explosive limits
Vapour pressure

Vapour density

Relative density(water = 1)
Water solubility

Partition coefficient: noctanol/water
Autoignition temperature
Decomposition temperature

Viscosity
	Colorless transparent liquid

/

/

/

-48～-52℃

114°C

35.0°C

/

Flammable.
3.4%～70%

20℃时1.54kPa

>1

0.97

Miscible

/
/
/
/


SECTION 10  STABILITY AND REACTIVITY

	Reactivity: /

Chemical stability: Stable under controlled storage conditions provided material contains adequate stabiliser / polymerisation inhibitor. Storage at higher temperatures and long term storage may result in polymerisation with solidification.  In larger quantities e.g. 200 l drums, this may result in the generation of heat (exotherm) which may release highly irritating boiling vapour.
Possibility of hazardous reactions: The substance may polymerize under the influence of heat, oxidizers, peroxides and light. Reacts violently with oxidants. Attacks many plastics.
Conditions to avoid: Heat, flames and sparks.
Incompatible materials: Incompatible with strong acids, acid chlorides, acid anhydrides, oxidising and reducing agents..
Hazardous decomposition products: Carbon dioxide (CO2), carbon monoxide (CO), other pyrolysis products typical of burning organic material.


SECTION 11  TOXICOLOGICAL INFORMATION

	Information on the likely routes of exposure: Inhaled, Ingestion, skin, eyes.
Symptoms related to the physical, chemical and toxicological characteristics: /
Acute health effects
Inhalation of vapours or aerosols (mists, fumes), generated by the material during the course of normal handling, may produce toxic effects.  Toxic effects may result from the accidental ingestion of the material. The material can produce chemical burns within the oral cavity and gastrointestinal tract following ingestion. Skin contact with the material may be harmful; systemic effects may result following absorption. The material can produce chemical burns within the oral cavity and gastrointestinal tract following ingestion. The material can produce chemical burns to the eye following direct contact. 
Chronic health effects: Repeated or prolonged exposure to corrosives may result in the erosion of teeth, inflammatory and ulcerative changes in the mouth and necrosis (rarely) of the jaw. Bronchial irritation, with cough, and frequent attacks of bronchial pneumonia may ensue. Gastrointestinal disturbances may also occur. Chronic exposures may result in dermatitis and/or conjunctivitis. 
Numerical measures of toxicity (such as acute toxicity estimates):  
Inhalation (Mouse) LC50: 2000mg/m3/2H


SECTION 12  ECOLOGICAL INFORMATION

	Toxicity: 
ENDPOINT

TEST DURATION (HR)

SPECIES

VALUE

LC50

96

Fish

1.53mg/L

EC50

48

Crustacea

3.36mg/L

EC50

96

Algae or other aquatic plants

19.300mg/L

NOEC

96

Fish

3.16mg/L

Persistence and degradability: Water/Soil: LOW. Air: LOW.
Bioaccumulative potential: LOW (LogKOW = 0.38)
Mobility in soil: LOW (KOC = 1.325)

Other adverse effects: /


SECTION 13  DISPOSAL CONSIDERATIONS

	Disposal methods: Recycle wherever possible. Consult manufacturer for recycling options or consult local or regional waste management authority for disposal if no suitable treatment or disposal facility can be identified. Dispose of by: burial in a land-fill specifically licenced to accept chemical and / or pharmaceutical wastes or Incineration in a licenced apparatus (after admixture with suitable combustible material). Decontaminate empty containers. Observe all label safeguards until containers are cleaned and destroyed.


SECTION 14  TRANSPORT INFORMATION

	UN number: 2929.
UN proper shipping name: TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S.
Transport hazard class(es): 6.1+3.
Packaging group: II.
Environmental hazards: /

Special precautions for user: / 


SECTION 15  REGULATORY INFORMATION

	Regulations：This safety data sheet is in compliance with the following national standards: GB/T16483-2008, GB13690-2009, GB18218-2009, GB15258-2009, GB6944-2012, GB190-2009, GB/T191-2008, GB12268-2012, GA57-1993, GB/T 15098-2008, GBZ 2-2007 as well as the following national regulations: Dangerous Goods Transport Administrative Regulation, Dangerous Chemicals Safety Administrative Regulation.


SECTION 16  OTHER INFORMATION

	References
	“Model Regulations on the Transport of Dangerous Goods”
“The Globally Harmonized System of Classification and Labelling of Chemicals”

	Form Date
	03-Apr-2019


Note 1: When products contain two or more hazardous substances, Safety Data Sheets should be prepared based on the risk of the mixture.

Note 2: Manufacturer / supplier should ensure the correctness of the information contained in the safety data sheets, and updated in a timely manner.

Note 3: As a result of product features without the existence of certain information (such as boiling point does not exist for the solid) in the table with "/" logo.
