Material safety data sheet

Section 1.  Product and Company Identification

1.1 Product Identification

Chemical name: 1,1',1"-Nitrilotri-2-propanol

Trade name: Triisopropanolamine 85% / TIPA 85%

Structural Formula: [CH3CH(OH)CH2]3N

CAS Number: 122-20-3

1.2 Company Identification

Manufacturers/Supplier: Shijiazhuang City Horizon Chemical Industry Co.,Ltd

Address:12th Floor of DongSheng Plaza,No 158 of PingAn North Street,Shijiazhuang City,Hebei Province,China

Tel: 86- 311- 66178338

Fax: 86- 311- 66178438

www.horizonadmixtures.com

Email:info@horizonadmixtures.com

1.3 EMERGENCY TELEPHONE NUMBER: 86- 311- 66178338

Section 2 Information on ingredients

| Chemical Entity | CAS No. | Proportions (%) |
| --- | --- | --- |
| 1, 1’, 1’’-Nitrilotri-2-PROPANOL | [122-20-3] | ≥85 |

Section 3 Hazards identification

Eye Contact: May cause moderate eye irritation. May cause moderate corneal injury.

Skin Contact: Prolonged contact may cause slight skin irritation with local redness. Repeated contact may cause skin burns. Symptoms may include pain, severe local redness, swelling, and tissue damage. May cause more severe response if skin is abraded (scratched or cut). May cause more severe response on covered skin (under clothing, gloves).

Skin Absorption: Prolonged skin contact is unlikely to result in absorption of harmful amounts.

Inhalation: At room temperature, exposure to vapor is minimal due to low volatility. Vapor from heated material may cause respiratory irritation and other effects.

Ingestion: Low toxicity if swallowed. Small amounts swallowed incidentally as a result of normal handling operations are not likely to cause injury; however, swallowing larger amounts may cause injury. Swallowing may result in gastrointestinal irritation.

Aspiration hazard: Based on physical properties, not likely to be an aspiration hazard.

Section 4 Emergency and first aid procedure

Eye: Immediately flush eyes in clear running water for at least 15 minutes, lifting upper and lower lids periodically. Get medical consultant immediately and preferable an eye specialist.

Skin: Wash off in flowing water.

Ingestion: Seek medical consultant. Do not vomit unless directed to do so by a medical personnel.

Inhalation: Remove the patient to fresh air.

Section 5 Fire-fighting measures

Flash point: Above 160°C

Flammability limits:

LFT: Not determined.

UFT: Not determined.

Extinguishing Media: Water fog or fine spray, carbon dioxide, dry chemical, foam. Alcohol resistant foams are preferred if available. Do not use direct water stream. Will spread fire.

Fire fighters wear positive-pressure, self-contained breathing apparatus and protective fire-fighting clothing.

Section 6 Leakage hazard data

The fire source should be removed first and keep the room ventilated. People dealing with the leakage should be protected as the section 5 has said and use enough sodium acid sulfate to cover the leakage. Then saturate with alkali. The waste liquid should be collected and put in open containers.

Section 7 Handling and storage

Handle with care. The container should be protected from attack, high pressure, collapse; friction and is not suitable to roll in the case. Labels outside the container should be kept in place. All the related electronic equipment should conform to certain fire standards. Most important of all, the outer package should not be damaged.

Store sealed in a cool, dry, well ventilated area away from acids, alkali, oxidizers, toxic or corrosive materials, combustible or explosive materials, long exposure to the sunlight. Do not roll containers. Stored in steel containers. Avoid to contact with copper or copper containing metals.  When the freezing point of TIPA is low , e.g. 15% of water content, aluminum containers is not suitable.  The container and its related equipment should be heated, insulated around 100℃. Use centrifugal pump to feed TIPA and seal the closure mechanically with TEFLON resin or asbestos fillings.

Section 8 Exposure controls or personal protection

Additional information about design of Technical systems:

Properly operating chemical fume hood designed for hazardous chemicals and having an average face velocity of at least 100 feet per minute.

8.1 Control parameters

Components with critical values that require monitoring at the workplace:

Not required.

Additional information: No data

8.2 Exposure controls

Personal protective equipment

General protective and hygienic measures:

The usual precautionary measures should be adhered to in handing the chemical.

Keep away from foodstuffs, beverages and food.

Instantly remove and soiled impregnated garments.

Wash hands during breaks and at the end of the work.

Avoid contact with the eyes.

Avoid contact with the eyes and skin.

Maintain and ergonomically appropriate working environments.

Breathing equipment: Use breathing protection with high concentrations.

Protection of hands:

Check protective gloves prior to each use for their proper condition.

The selection of the suitable gloves does not only depend on the material, but also on further marks of quality and varies from manufacturer to manufacturer.

Material of gloves: Impervious gloves

Penetration time of glove material: Not determined

Eye protection: Safety glasses

Body protection: Protective work clothing.

Section 9 Physical and chemical characteristics

| Appearance |  |  |
| --- | --- | --- |
| Physical State | Liquid. |  |
| Color | Colorless to yellow |  |
| Odor | Slightly ammoniacal |  |
| Odor Threshold | No test data available |  |
| PH | No test data available |  |
| Melting Point | Not determined |  |
| Freezing Point | 5 °C Literature (Supercools; freezing point may therefore vary) |  |
| Boiling Point (760 mmHg) | 104 °C Literature. |  |
| Flash Point - Closed Cup | none below boiling point |  |
| Flash Point - Open Cup | 160 °C Cleveland Open Cup |  |
| Evaporation Rate (Butyl | No test data available |  |
| Acetate = 1) |  |  |
| Flammability (solid, gas) | Not applicable to liquids |  |
| Flammable Limits In Air | Lower: Not determined |  |
|  | Upper: Not determined |  |
| Vapor Pressure | < 0.01 mmHg Literature |  |
| Vapor Density (air = 1) | Not determined |  |
| Specific Gravity (H2O = 1) | 1.027 25 °C/4 °C Literature |  |
| Solubility in water (by | 100 % Literature Completely miscible in all proportions |  |
| weight) |  |  |
| Partition coefficient, n- | -0.015 Measured |
| octanol/water (log Pow) |  |
| Autoignition Temperature | Not determined |
| Decomposition | No test data available |
| Temperature |  |
| Dynamic Viscosity | 240 cps @ 25 °C Literature |  |
| Kinematic Viscosity | No test data available |

Section 10 reactivity and stability data

Stability: not stable under long exposure to high temperature. If uncovered in open air, the product will absorb water and CO2.

Incompatibility (materials to avoid): as stipulated in Section 7

Hazardous decomposition or byproducts: none.

Section 11 Toxicity data

TIPA belongs to organic alkali weak corrosive chemicals.

Repeated and prolonged exposure may result in certain skin inflammation.

Ingestion maybe have toxic results.

Eye, nose, skin contact with vapor may irritate. Prolonged contact will affect the eyesight.

Section 12 Biological data

No official report is available. However, this product hasn’t apparent affection on the environment if the density is quite low. Otherwise, it will produce certain adverse influence on the water source, land, and aquatic organisms.

Section 13 Waste disposal method suggestion

TIPA and the packing materials can be recycled. Those failing to be recycled can be dealt with according to Section 6.

Section 14 Transportation data

14.1 UN-Number

ADR, IMDG, IATA                  None

14.2 UN proper shipping name

ADR, IMDG, IATA                  None

14.3 Transport hazard class (es)

ADR, IMDG, IATA                  None

14.4 Packing group

ADR, IMDG, IATA                  None

14.5 Environmental hazards:          Not applicable.

14.6 Special precautions for user:      Not applicable

14.7 Transport/Additional information: Not dangerous according to the above specifications.

Section 15 Legal requirements

For TIPA 85%, it belongs to organic weak-corrosive chemicals. The products should not be labeled with danger mark.

Section16 Other information

The information contained herein is believed to be accurate. However, it is provided solely for the customer consideration, investigation and verification. Manufacturer hereby specifically disclaims any and all warranties, expressed or implied, regarding the accuracy and completeness of such information, and makes no representations with respect thereto.